# PipeWire LE Audio Minimal Implementation

## Introduction

This document explains the minimal set of PipeWire and SPA BlueZ5 files required to implement an LE Audio–only audio pipeline.

The flow chart:

User Application (PCM)

PipeWire Graph (routing, format negotiation)

SPA BlueZ5 Plugin entry — plugin.c

BlueZ D-Bus handlers — bluez5-dbus.c

(listens for LE Audio: BAP/PACS/ASE interfaces via D-Bus)

LE Audio Device Manager — bluez5-device.c

(creates PipeWire nodes, selects LC3 configuration, manages ASE states)

Capture Node — media-source.c       Playback Node — media-sink.c

(PCM → LC3 encode)                 (LC3 decode → PCM)

LC3 Codec — bap-codec-lc3.c

(with capability map — bap-codec-caps.h)

LC3 frames (PDUs)                      LC3 frames (PDUs)

ISO Transport — iso-io.c / iso-io.h

(Bluetooth ISO sockets: send/receive LE Audio PDUs)

BlueZ Kernel + Bluetooth Controller

(ISO radio link to LE Audio device)

## Pipeline Flow Overview

LE Audio flow in PipeWire involves multiple stages:

1. Device Discovery — BlueZ detects LE Audio capable devices and exposes BAP/PACS/ASE interfaces via D-Bus.

2. Device Connection — PipeWire's BlueZ5 plugin learns about the device and creates internal objects.

3. Stream Configuration — Codecs (LC3) and ASE parameters are negotiated.

4. Data Streaming — LC3 frames encoded/decoded → transmitted/received via ISO sockets.

Each stage uses specific files from the minimal implementation.

## plugin.c

Purpose: Acts as the entry point for the BlueZ5 SPA plugin. It registers and initializes the plugin in PipeWire.

Why Required: Without this file, PipeWire cannot load the BlueZ5 plugin, and LE Audio device discovery will not begin.

Stage Used: Startup Stage – before any device discovery begins.

Triggered From: PipeWire core loads the plugin via SPA factory mechanism.

Output Usage: Initializes D-Bus monitors → triggers `bluez5-dbus.c`.

## bluez5-dbus.c

Purpose: Handles all communication with BlueZ over D-Bus. Receives events like InterfacesAdded, PropertiesChanged.

Why Required: This is the ONLY file that understands BlueZ LE Audio interfaces such as BAP, PACS, ASE.

Stage Used: Device Discovery + Device Connection.

Triggered From: plugin.c initializes D-Bus monitor which triggers callbacks inside this file.

Output Used In:

- `bluez5-device.c` for creating LE Audio device objects.

- `media-source.c` and `media-sink.c` when streams transition to streaming state.

## dbus-monitor.h

Purpose: Header declaring APIs for D-Bus event monitoring.

Why Required: Allows other modules to receive D-Bus event callbacks.

Defines:

- Initialization functions

- Signal handler prototypes

- Event structure definitions

Stage Used: Included by device, transport, and dbus modules.

Triggered From: Used when D-Bus monitoring is set up.

Output Used In:

- dbus-monitor.c implementation

- bluez5-device.c

- bluez5-transport.c

## dbus-monitor.c

Purpose: Monitors BlueZ D-Bus signals.

Why Required: Notifies PipeWire when devices or streams change.

Also Detects:

- Device added/removed

- ASE state changes

- Transport creation/removal/updates

Stage Used: Runtime monitoring.

Triggered From: Started by bluez5-dbus.c during backend initialization.

Output Used In:

- bluez5-device.c for device/ASE updates

- bluez5-transport.c for transport state

- Media node setup indirectly

## bluez5-device.c

Purpose: Represents a Bluetooth LE Audio device inside PipeWire.

Why Required: Responsible for parsing device capabilities, creating media nodes, configuring ASEs, handling stream transitions.

Stage Used: Device Connection → Stream Configuration.

Triggered From: bluez5-dbus.c when it detects new LE Audio device interfaces.

Output Used In:

- Creating playback node → calls `media-sink.c`

- Creating capture node → calls `media-source.c`

- Selecting codec parameters → uses `bap-codec-caps.h`

## bap-codec-caps.h

Purpose: Defines LC3 codec capabilities such as supported sampling rates, frame duration, bitrates.

Why Required: Needed to negotiate LC3 parameters with BlueZ and configure ASE.

Stage Used: Stream Configuration.

Triggered From: bluez5-device.c when selecting LC3 configuration.

Output Used In: LC3 encoder/decoder initialization in `bap-codec-lc3.c`.

## bap-codec-lc3.c

Purpose: Implements the LC3 encoder and decoder.

Why Required: LE Audio uses LC3 exclusively. Without this file, no audio can be encoded or decoded.

Stage Used: Streaming Stage.

Triggered From:

- `media-sink.c` (encoding before sending ISO frames)

- `media-source.c` (decoding after receiving ISO frames)

Output Used In: Output LC3 frames → fed to `iso-io.c`. Decoded PCM → PipeWire graph.

## media-sink.c

Purpose: Implements the PipeWire playback node for LE Audio.

Why Required: Encodes PCM from applications into LC3 → sends via ISO.

Stage Used: Playback Streaming Stage.

Triggered From: bluez5-device.c when ASE (playback) becomes active.

Output Used In: ISO transport (`iso-io.c`).

## media-source.c

Purpose: Implements PipeWire capture node for LE Audio.

Why Required: Receives ISO frames → decodes LC3 → outputs PCM.

Stage Used: Capture Streaming Stage.

Triggered From: bluez5-device.c when capture ASE becomes active.

Output Used In: Applications via PipeWire graph.

## iso-io.c and iso-io.h

Purpose: Implements ISO transport layer for LE Audio: sending and receiving ISO PDUs using Bluetooth kernel sockets.

Why Required: ISO transport is mandatory for LE Audio. This handles sequence numbers, timestamps, PDUs.

Stage Used: Streaming Stage.

Triggered From: media-source.c and media-sink.c.

Output Used In: Bluetooth controller for radio transmission.

## player.c and player.h

Purpose: Provides precise timing and scheduling for transmitting LC3 frames.

Why Required: LC3 frames must be delivered every 7.5ms/10ms accurately.

Stage Used: Streaming Stage.

Triggered From: iso-io.c and media-sink.c.

Output Used In: Frame pacing and synchronization.

## rate-control.h

Purpose: Adjusts bitrate dynamically and handles drift compensation.

Why Required: Maintains audio sync between controller and host.

Stage Used: Long-running streaming.

Triggered From: player.c.

Output Used In: LC3 frame adaptation.

## bt-latency.h

Purpose: Defines latency models for LE Audio.

Why Required: Required when configuring ASE to match expected device latency.

Stage Used: Stream Configuration.

Triggered From: bluez5-device.c.

Output Used In: ASE QOS setup.

## decode-buffer.h

Purpose: Implements PCM buffering for decoding stage.

Why Required: Incoming ISO packets need buffer for jitter handling.

Stage Used: Capture Streaming.

Triggered From: media-source.c.

Output Used In: PipeWire PCM output.

## defs.h

Purpose: Provides shared constants, macros, and enums.

Why Required: Used across entire BlueZ5 LE Audio stack.

Stage Used: All stages.

Triggered From: Included everywhere.

## SPA Core Files

SPA framework files required:

1. spa/utils/*.h — fundamental utility macros, memory, lists.

2. spa/param/audio/ — definitions of PCM formats, channels.

3. spa/support/log.c/h — logging backend.

4. spa/support/system.c — poll, timer, event loop.

5. spa/node/ — base class for PipeWire nodes.

Why Required: LE Audio nodes depend on SPA node, logging, timers, audio params.

Stage Used: Node creation → Streaming.

Triggered From: media-source.c and media-sink.c.

Output Used In: Entire node lifecycle.